DIGITAL DESIGN

&

COMPUTER ORGANISATION

MINI PROJECT

TITLE: SMART WATER LEVEL CONTROLLER USING FSM

TEAM MEMBERS:

- ABDUL MUHEETH K (PES1UG24CS550)

- ANIRUDH M (PES1UG24CS556)

- PRAVEEN (PES1UG25CS832)

PROBLEM STATEMENT

Project title: Smart Water Level Controller using Finite State Machine (FSM) with Verilog implementation.

Problem statement:
Design a digital controller that automatically manages a water pump to maintain tank water level using three binary level sensors (Low, Mid, High). The controller must:

- Turn the pump ON when the tank is empty or below the desired level.

- Keep the pump ON while the tank fills up until the high sensor is reached.

- Turn the pump OFF when the high level is reached to prevent overflow.

- Respond to falling water level by re-enabling the pump when necessary.

- Be implemented as a synchronous finite state machine with an asynchronous reset.

- Provide optional features: manual override, LED indicators, alarm on overflow/fault, and basic debouncing or filtering of sensor inputs.

Inputs: S0 (low/empty sensor), S1 (mid sensor), S2 (high sensor), clk, reset, (optional manual_on, manual_off).

Outputs: pump (drive to pump relay/driver), state_leds (2-bit state indicator), alarm (optional).

Performance constraints:

- Synchronous design with a single clock domain.

- Robust to contact bounce: add debouncing or simple digital filtering.

- Safe default: on reset, pump should be ON to avoid water starvation (or design choice: OFF — but this project uses ON on reset).

SOLUTION

Overview:

- States (CS1 CS0): 00 = EMPTY, 01 = HALF, 10 = FULL.

- Inputs: S0 (low), S1 (mid), S2 (high).

- Outputs: pump (1 = ON, 0 = OFF).

- Storage: two D flip-flops store CS1 (MSB) and CS0 (LSB).

- Next states (NS1 NS0): 00 = EMPTY, 01 = HALF, 10 = FULL.

Truth table:

- From EMPTY (00) state

| CS1 CS2 | S2 S1 S0 | NS1 NS0 | Pump |
| --- | --- | --- | --- |
| 00 | 000 | 00 | 1 |
| 00 | 001 | 00 | 1 |
| 00 | 010 | 01 | 1 |
| 00 | 011 | 01 | 1 |
| 00 | 100 | 01 | 1 |
| 00 | 101 | 01 | 1 |
| 00 | 110 | 01 | 1 |
| 00 | 111 | 01 | 1 |

- From HALF (00) state

| CS1 CS2 | S2 S1 S0 | NS1 NS0 | Pump |
| --- | --- | --- | --- |
| 01 | 000 | 00 | 1 |
| 01 | 001 | 01 | 1 |
| 01 | 010 | 01 | 1 |
| 01 | 011 | 01 | 1 |
| 01 | 100 | 10 | 1 |
| 01 | 101 | 11 | 1 |
| 01 | 110 | 11 | 1 |
| 01 | 111 | 11 | 1 |

- From FULL (10) state

| CS1 CS2 | S1 S2 S3 | NS1 NS2 | Pump |
| --- | --- | --- | --- |
| 10 | 000 | 01 | 0 |
| 10 | 001 | 01 | 0 |
| 10 | 010 | 01 | 0 |
| 10 | 011 | 01 | 0 |
| 10 | 100 | 10 | 0 |
| 10 | 101 | 10 | 0 |
| 10 | 110 | 10 | 0 |
| 10 | 111 | 10 | 0 |

Boolean Expression:

Next-state bits (use these to drive D inputs of the flip-flops):

NS1 = (S2 & CS0 & ~CS1) | (S2 & ~CS0 & CS1)

NS0 = (~CS1 & ~CS0 & (S1 | S2))

| (~CS1 & CS0 & S0 & ~S2)

| ( CS1 & ~CS0 & ~S2)

Pump = ~CS1

Equivalent compact forms used during derivation:

- NS1 = S2 & (CS0 ^ CS1)

- NS0 = (CS==00 & (S1|S2)) | (CS==01 & S0 & ~S2) | (CS==10 & ~S2)

Digital Circuit:

Flowchart of working:

Working Diagram:

FILLING IN THE TANK

S2

S1

S0

S2

S1

S0

Stage 0                                                                                 Stage 1

Sensors: S2=0 S1=0 S0=0                                                   Sensors: S2=0 S1=0 S0=1

FSM state: EMPTY (00)                                                       FSM state: EMPTY(00)

S2

S1

S0

S2

S1

S0

Pump: ON (1)                                                                      Pump: ON (1)

Stage 3                                                                                       Stage 2

Sensors: S2=1 S1=1 S0=1                                                      Sensors: S2-0 S1=1 S0=1

FSM state: HALF (01) -> FULL (10)                                FSM state: EMPTY (00) -HALF (01)

PUMP: OFF (0)                                                                         PUMP: ON (1)

WATER CONSUMPTION

S2

S1

S0

S2

S1

S0

Stage 4                                                                             Stage 5

Sensors: S2=0 S1=1 S0=1                                             Sensors: S2=0 S1=0 S0=1

FSM state: FULL (10)-> HALF (01)                                  FSM state: HALF (01) -> EMPTY(00)

S2

S1

S0

PUMP: ON (1)                                                                   PUMP: ON (1)

Stage 6

Sensors: S2=0 S1=0 S0=0

FSM state: EMPTY (00) -> completely drain

PUMP: ON (1)

VERILOG IMPLEMENTATION

Verilog hardware file:

//====================================== // Smart Water Level Controller using FSM // Compatible with Icarus Verilog //======================================

module SmartWaterLevelController(

input clk,

input reset,

input S0, S1, S2,  // Sensors

output reg pump  // Pump control );

// State encoding (no typedef)
parameter EMPTY = 2'b00;
parameter HALF  = 2'b01;
parameter FULL  = 2'b10;

reg [1:0] current_state, next_state;

// Sequential block: state transition
always @(posedge clk or posedge reset) begin
    if (reset)
        current_state <= EMPTY;
    else
        current_state <= next_state;
end

// Combinational block: next state logic
always @(*) begin
    case (current_state)

EMPTY: begin
            if (S1 || S2)
                next_state = HALF;
            else
                next_state = EMPTY;
        end

        HALF: begin
            if (S2)
                next_state = FULL;
            else if (!S0)
                next_state = EMPTY;
            else
                next_state = HALF;
        end

        FULL: begin
            if (!S2)
                next_state = HALF;
            else
                next_state = FULL;
        end

        default: next_state = EMPTY;
    endcase
end

// Output logic (Moore type)
always @(*) begin
    case (current_state)
        EMPTY: pump = 1'b1; // Turn ON pump
        HALF:  pump = 1'b1; // Keep pump ON
        FULL:  pump = 1'b0; // Turn OFF pump
        default: pump = 1'b0;
    endcase
end

endmodule

Testbench:

//======================================

// Testbench for Smart Water Level Controller

// Compatible with Icarus Verilog

//======================================

`timescale 1ns/1ps

module tb_SmartWaterLevelController;

reg clk, reset;

reg S0, S1, S2;

wire pump;

// Instantiate DUT

SmartWaterLevelController dut (

.clk(clk),

.reset(reset),

.S0(S0), .S1(S1), .S2(S2),

.pump(pump)

);

// Clock generation

always #5 clk = ~clk;  // 10ns clock period

initial begin

// Dumping setup for GTKWave

$dumpfile("SmartWaterLevel.vcd");

$dumpvars(0, tb_SmartWaterLevelController);

// Initialize

clk = 0;

reset = 1;

S0 = 0; S1 = 0; S2 = 0;

#10 reset = 0;

// Test sequence

#10 S0=0; S1=0; S2=0;  // Empty

#20 S0=1; S1=0; S2=0;  // Low

#20 S0=1; S1=1; S2=0;  // Mid

#20 S0=1; S1=1; S2=1;  // Full

#20 S0=1; S1=1; S2=0;  // Dropping

#20 S0=1; S1=0; S2=0;  // Lower

#20 S0=0; S1=0; S2=0;  // Empty again

#20 $finish;

end

// Display states in numeric form

initial begin

$monitor("Time=%0t | S0=%b S1=%b S2=%b | State=%b | Pump=%b",

$time, S0, S1, S2, dut.current_state, pump);

end

endmodule

vvp output:

Gtkwave output: